Sadelco MiniMax M800

Frequency Range: 5 - 872 MHz		Features:

Power Out: Built-in battery provides

Standard Channel Plans			switchable 18V at F connector for

EIA (NTSC), HRC, IRC, AIR		powering downconverters

(VHF, UHF, FM), MMDS			C/N: Range > 50 dB with external

(PAL and Custom Channel Plans		preselector

available on request.)			C/N Accuracy: ± 2dB

Hum: 0.5 - 5%, ± 0.3%, detects all

Tuning Increments			hum up to 600 HZ

Frequency Mode:  125 KHz	                     Favorite Channels: 9 user

Channel Mode:  6, 7, or 8 MHz		Programmable Channels

Delta PA: Automatically calculates

Frequency Accuracy			and displays difference between

± 20 KHz				picture and audio levels of same

channel

IF Bandwidth			Audio: Adjustable volume, waterproof

280 KHz at 3 dB points			mylar speaker

Illumination: Switchable lights for

FM Selectivity			both LCD and Microammeter

Will distinguish 2 FM signals		Battery Test: Shows battery voltage

of similar amplitude	 		and elapsed time

300 Khz apart			Power Supply: Built-in NiCads provide

>16 hrs of normal intermittent use

Measurement Range			Electronic Shut-Off: User adjustable,

-20 to +40 dBmV (+40 to +100 dBuV)	                    5 min, 10 min or continuous

Digital Channel Measurement:

Amplitude Accuracy			Measures average power of QAM

± 1.0 dB at 77° F			signals

± 2.0 dB from 0° - 120° F

Dimensions

Auto Calibration			8.5" L x 4" W x 3" D

Each measurement computer		21 cm x 10 cm x 7.6 cm

corrected for level, attenuator

setting and temperature			Weight

Internal Calibrator: ± 0.25 dB		4.5 lbs (2Kg), including case

4.5 - 864 MHz white noise source	                     and batteries

Dual Display				Accessories Supplied

Digital LCD: Indicates channel,		Nylon Case: Part# CASE003

frequency and signal level in		AC 110V Charger/Adapter: Part# T60

dBmV (switchable to dBuV)		Nicad Battery Pack: Part# BAT05

Analog Microammeter: Indicates real	                     (15 Sub C Cells)

time signal level for easy antenna	                     Operator's Manual: Part# ST1003

orientation , ± 1.5 dB

Optional Accessories

Attenuator				StrandHook: Part# SH2

Range: 20 dB computer corrected		220/240 Charger/Adapter: Part# T60E

Input: 75 ohm, user replaceable F81

connector, BNC available on request.	                    Specifications Subject to Change Without Notice