# Imagess Satellite Receiver R4700

At a Glance:

- Commercial quality performance on NTSC, PAL, and SECAM signals

- AFC tracking circuit with continuous fine tuning ability to correct for LNB drift

- AGC tuning voltage for antenna peaking

- Two IF bandpass  filters 27 and 18 MHz

- Microprocessor controlled single conversion PLL RF tuning

- Stock with one multistandard frequency agile audio demodulator

General Specifications:

Voltage: 115-230 VAC  50-60 Hz

Current (Max): 20W

Size: 19”W x 12”D x 1.75”H

(48.26W x 30.48D x 4.44H) cm

Video Output:

C/L Delay (Video Out): 60 n-sec

C/L Delay (Comp Out): 25 n-sec

Short Time Distortion: 5 IRE

Differential Gain: 4.0%

Differential Phase: 4.0 deg

Signal to Noise Ratio: 60 dB

Frequency Response: +/- .75 dB

Audio Input:

Stock (agile): 5.0 - 8.5 MHz

CAD620 (agile): 5.0 – 8.5 MHz

Audio Output:

Frequency Response: 1.5 dB

Output Impedance: 600 ohm balanced

Output Level: +18 dBm

THD: 1.5%

RF Characteristics:

Frequency Range: 950 – 2050 MHz

First IF: 479.5 MHz

Input Level: -20 to -60 dBm

Wide IF Bandwidth: 27 MHz

Narrow IF Bandwidth: 18 MHz

Tuning (dial): 1.0 MHz

AFC Capture Range: +/- 3 MHz

AGC Dynamic Range: 40 dB

Signal Meter: Five Segment LED

Static Threshold: < 7.0 dB