# General Instruments Model STS-450M, NTSC Frequency Agile Triple Modulator

Features

- SPACE EFFICIENT: 3 MODULATORS IN 1 RACK UNIT (1.75 INCHES)

- SINGLE COMBINED RF OUTPUT

- FREQUENCY AGILE FROM 50 MHz TO 450 MHz

- INDEPENDENT COMPOSITE IF LOOP FOR EACH MODULATOR

- FCC COMPLIANT

Description

The STS-450M is a cost effective headend product containing three independent frequency agile modulators in a one rack unit (1.75 inch) space. The STS-450M is designed to meet the economical and space needs of SMATV and cable system operators.

All adjustments, with the exception of stereo enable, can be made via convenient front panel controls. Each modulator has independent controls for channel selection; FCC frequency offset adjustment, video depth of modulation, audio deviation, audio/video carrier ratio and RF output level. LED indicators on the front panel warn of video and audio overmodulation conditions for each modulator.

Each modulator in the STS-450M accepts monaural baseband audio and can be internally configured to accept a baseband (MPX) stereo encoded input signal.

Specifications

| RF |  |
| --- | --- |
| Frequency Range | 50 MHz to 450 MHz |
| Output configuration | 3 channels combined |
| Output level | +50 dBmv /channel min. |
| Output level range | 0 to -10 dB |
| A/V carrier level range | -12 to -20 dBc |
| Spurious output | -60 dBc max @ max RF output |
| C/N ratio (normalized to 4.0 MHz) |  |
| In-band | 60 dB min |
| Out-of-band | 70 dB min |
| Output return loss | 14 dB min within channel |
| Frequency accuracy | ± 3 kHz from nominal |
| VIDEO |  |
| Input level range | 0.7 Vp-p min to 1.4 Vp-p max @87.5% modulation. |
| Input impedance | 75W |
| Input return loss | 20 dB min (10 Hz to 4.2 MHz) |
| Signal to noise ratio | >59 dB (luminance wghtd) |
| Differential gain | ± 3% max @ 87.5% mod. |
| Differential phase | ± 5° max @ 87.5% mod. |
| Frequency response | ± 1 dB max (25 Hz to 3.58 MHz) |
| Chrominance to Luminance delay | ± 50 ns maximum |
| K-factor | 4% maximum |
| AUDIO |  |
| Input level | 300 mV p-p max. |
| Input impedance | 10 kW unbalanced |
| Frequency response | ± 2.0 dB max (30 Hz to 13 kHz) |
| Preemphasis | 75 ms |
| Harmonic distortion | 3% maximum (30 Hz to 13 kHz, @ ± 25 kHz dev.) |
| Subcarrier frequency | 4.5 MHz ± 500 Hz |
| GENERAL |  |
| AC power | 100 to 135 Vrms, 60 Hz, 30 W maximum |
| Operating temperature | +20° to +40° C |
| Dimensions | 19" W x 1.75" H x 9" D |